This code turns Mega into a smart, reliable, field data logger. It gathers data from sensors (mq2, mq4, pressure sensor, DHT22), saves it permanently to an SD card, and then sends it to your ESP32 bridge using a robust, error-checking Bluetooth link.

Code Nature:

The Data Source: This is the main brain. It reads all the sensors

Reliable Sender: This code doesn't just Serial.print data over Bluetooth and hope it arrives. It uses a custom protocol:

- It wraps every piece of data (live or from the CSV) in a special "frame" with a sequence number and an error-check (CRC).

- It sends the frame and then waits for the ESP32 to send back an "ACK" (acknowledgment) with the same sequence number.

- If it doesn't get the ACK, it assumes the packet was lost and re-sends it up to 5 times. This makes the data transfer very robust.

On-Demand History: It doesn't constantly stream the old data. It waits patiently until the ESP32 sends it a command:

- "REQ": "Send me everything from the beginning."

- "FAST": "Just send me the last 2KB of data." (For latest 15 readings)

Main Functions

- Sampling (Every SAMPLE_INTERVAL_MS):

  - Reads MQ2 (Ethylene), MQ4 (Methane), DHT22, and BMP280 sensors.

  - Converts sensor values into PPM and standard units.

- Data Logging (Dual-Mode):

  - CSV Log: Appends a new, human-readable line to datalog.csv on the SD card for easy analysis in Excel.

- Live Data Push:

  - Sends every new SAMPLE_INTERVAL_MS, sample to the ESP32 using the reliable frame system (which the ESP32 then posts to your server's /pong endpoint).

- CSV History Dump:

  - When it gets a "REQ" or "FAST" command, it opens datalog.csv.

  - It reads the file (either from the start or just the end) in small, 20-byte chunks.

  - It sends each chunk using the reliable frame system (which the ESP32 posts to /csv_chunk).

  - When the file is done, it sends a special "END" frame so the ESP32 knows the transfer is complete.

Serial Monitor shows:

- [DATA] ts=... mq2_ppm=... (Shows you the full sensor reading)

- [CSV] appended (Confirms data was saved to the SD card)

- [BLE] REQ → stream ALL datalog.csv (Tells you when a history dump starts)

- [BLE] send failed → abort CSV stream (Warns you if the BLE connection drops during a transfer)