Graph Representation:

- Y-axis (PPM): Parts per million; higher ppm = more target gas.

- Pipeline: ADC → Vout → Rs = RL·(5−Vout)/Vout → (Rs/R0) → PPM.

- Terms: Rs = sensor resistance (drops with more gas); R0 = clean-air baseline (from calibration); RL = fixed load resistor.

- Meaning: Rs/R0 shows overall gas effect (mixture). PPM is a per-gas estimate using a fitted curve.

- Curves used:

MQ-2 ≈ 50·(Rs/R0)^−2.3 (hydrocarbons/Ethylene);

MQ-4 ≈ 1000·(Rs/R0)^−2.95 (Methane).

- Reading the graph: More gas → lower Rs/R0 → higher ppm. Peaks = gas bursts; flat/low = clean or stable air.