Report on NFC-Based Low-Power Sensor Data Reading System

1. Introduction / Background

In our project, we use an MCU (Arduino UNO or Mega) connected with multiple sensors, MQ-2, MQ-4, and DHT22 along with a microSD card module.
The system periodically measures sensor values (every 10 minutes) and stores them in a CSV file on the SD card for data logging and later analysis.

However, continuous wireless transmission using Bluetooth or Wi-Fi modules was found to be power-hungry, making them unsuitable for battery-powered or energy-harvesting systems (like solar or electromagnetic recovery setups).
To achieve ultra-low-power real-time data access, we adopted an NFC-based communication interface using the PN532 NFC Module V3.

This module operates at 13.56 MHz (per ISO/IEC 14443 A/B standards) and supports NFC peer-to-peer, card emulation, and reader modes.
It enables reading of latest sensor values directly from the MCU memory without requiring continuous high-power wireless connectivity.

2. Power Efficiency Motivation

According to the PN532 module datasheet (NXP Semiconductors, 2021):

| Parameter | Typical Value | Description / Reference |
| --- | --- | --- |
| Operating voltage | 2.7 – 5 V | Compatible with Arduino UNO/Mega logic levels |
| Operating current | ≈ 60 mA (typical) / 150 mA (max) | Active RF field during data transmission |
| Standby current (RF field on) | 2–45 µA | Ultra-low standby consumption |
| Frequency | 13.56 MHz | ISO/IEC 14443 A/B and FeliCa compliant carrier (A global standard for contactless smart cards operating at 13.56 MHz). |
| Communication interface | I²C / SPI / UART | Selectable by DIP switch on V3 board |

In comparison:

- Bluetooth (HC-05 / HM-10): 25–40 mA continuous active current

- Wi-Fi (ESP8266 / ESP32): 70–170 mA transmit current peaks

- NFC (PN532): < 100 mA only during tag read, then drops to < 10 µA idle

Hence, NFC reduces average current draw when data access is infrequent (once per few minutes), making it ideal for low-duty-cycle environmental sensor nodes powered by small batteries or energy harvesters.

3. Overview of NFC Technology

3.1 Principle of Operation

Near-Field Communication (NFC) is a short-range (≤ 10 cm) wireless technology standardized under ISO/IEC 18092 and ECMA-340 (A global standard for contactless smart cards operating at 13.56 MHz).
It operates at 13.56 MHz, enabling bidirectional data exchange between:

- a Reader/Writer (Initiator)

- and a Tag/Target (Responder).

NFC supports three operation modes:

- Reader/Writer Mode, Reads/writes passive tags (Type A/B/FeliCa).

- Peer-to-Peer (P2P) Mode, Two NFC devices exchange data (Active ↔ Passive).

- Card Emulation Mode, NFC device behaves like a contactless smart card.

Typical data rates: 106 kbps – 424 kbps, with modulation via amplitude-shift keying (ASK).

Reference: PN532 Datasheet

4. PN532 V3 Module – Description and Features

4.1 Hardware Overview

The PN532 V3 module integrates NXP’s PN532 transceiver IC, an RF antenna, and level-shifting logic for Arduino compatibility.
It exposes SPI, I²C, and UART interfaces selectable by onboard DIP switches.

Key specs from the datasheet:

| Feature | Description |
| --- | --- |
| Chipset | NXP PN532 (ISO/IEC 14443 A/B, 18092) |
| Frequency | 13.56 MHz ± 7 kHz |
| Interface | UART / I²C / SPI selectable |
| Voltage | 3.3 – 5 V tolerant |
| Current | 60 mA active RF, < 10 µA standby |
| Range | Up to 5 cm (depends on antenna & field strength) |

The module’s onboard DIP switch configures the interface:

- SW1 = ON, SW2 = OFF → UART

- SW1 = OFF, SW2 = ON → I²C

- SW1 = OFF, SW2 = OFF → SPI

5. Implementation for Sensor Data Access

5.1 System Architecture

- Sensors → analog/digital inputs to MCU.

- Arduino logs readings to datalog.csv on the SD card every 10 min.

- PN532 connects via SPI/I²C to the same MCU.

- Smartphone or another NFC device reads the latest value from Arduino memory when tapped.

5.2 Method 1: Using a Single NFC Module and NFC-Enabled Mobile Phone

Concept:
The PN532 operates in Card Emulation Mode (Target mode).
The Arduino continuously updates a small NDEF message containing the latest reading.
When a phone is brought near the module, it reads this message as an NFC tag.

Data Flow:

- MCU measures sensors and stores readings:

- Example readings: Temp=27.3°C, Hum=51%, MQ2=143 ppm, MQ4=210 ppm

- MCU formats this as an NDEF Text record.

- PN532 stores and advertises the NDEF data in emulation mode.

- Phone’s NFC reader energizes the field → reads the NDEF payload.

- Android/iOS automatically shows the message.

Advantages:

- No pairing, no app required.

- Ultra-low power (RF only active during tap).

- Secure short-range (≤ 5 cm).

5.3 Method 2: Two NFC Modules (P2P Communication Between MCUs)

Concept:
Use one PN532 as Initiator (Reader/Controller) and another as Target (Sensor Node).
This leverages the Peer-to-Peer Mode defined by NFCIP-1 (ISO/IEC 18092).

Hardware Setup:

- Node A (Sensor Unit): Arduino + Sensors + PN532 (Target)

- Node B (Receiver Unit): Another MCU + PN532 (Initiator)

Process:

- Sensor Node periodically prepares the latest readings.

- When Receiver Node comes near (within 5 cm), it establishes an NFC P2P link.

- The Initiator requests data using “DEP (Data Exchange Protocol)” frames.

- The Target sends back formatted sensor readings (text or binary).

- Receiver displays data on Serial Monitor or LCD.

Advantages:

- Works without smartphones.

- Secure, device-to-device exchange.

- Moderate range (≤ 5 cm).

- Data integrity via CRC16 (per ISO 14443 frame).

5.4 Method 3: Using PN532 as Reader to Poll Passive Tag

In alternate mode, the PN532 acts as a Reader, and a programmable NFC tag (MIFARE Ultralight/NTAG) can store recent readings.
Arduino can update tag memory via I²C at each interval; phone can read tag directly later.
This decouples power domains: the tag remains readable even if MCU is off.

5.5 If Mobile Phone Does Not Support NFC

- Option 1: Use another MCU + PN532 as Initiator (Method 2).

- Option 2: Employ a BLE module (HM-10) temporarily for diagnostics.

- Option 3: Retrieve logs manually via SD card or serial USB connection.

Thus, NFC acts as a primary low-power interface, with BLE/USB serving as fallback for non-NFC devices.

6. Power and Efficiency Comparison

| Interface | Typical Active Current | Standby | Range | Data Type | Power Efficiency |
| --- | --- | --- | --- | --- | --- |
| Wi-Fi (ESP8266) | 120 mA avg | 15 mA | > 10 m | High-speed TCP/IP | ❌ High power |
| Bluetooth (HM-10) | 30 mA | 5 mA | ≤ 10 m | UART stream | ⚠️ Medium |
| NFC (PN532 V3) | 60 mA active | < 100 µA | ≤ 5 cm | NDEF text/P2P | ✅ Very Low |

Reference:

- Elechouse PN532 Manual Sec. 3.2 & 6.3 (Electrical/Power).

- HC-05/ESP8266 datasheets (for comparison).

By using NFC for occasional data access, total energy consumption drops by > 90 % versus continuous Bluetooth/Wi-Fi transmission.

7. Conclusion

The integration of the PN532 V3 NFC module with the sensor-logging MCU system offers a technically efficient and power-optimized solution for real-time data retrieval in portable and autonomous devices.

- NFC’s low activation current (< 60 mA) and microamp-level standby drastically extend battery life.

- Card Emulation Mode enables instant “tap-to-read” access using any NFC-enabled smartphone.

- Peer-to-Peer Mode provides MCU-to-MCU data transfer without Internet or pairing.

- System remains fully functional even if phones lack NFC support (data available via SD or serial).

This approach aligns with modern low-power IoT design principles, leveraging short-range inductive coupling for minimal-energy, secure, and on-demand communication.

8. Key References

🧩 1️⃣ What the PN532 actually is

The PN532 chip is an NFC controller, not an NFC memory.
It has:

- A small RAM buffer (a few hundred bytes)

- A microcontroller core inside to handle NFC protocol (ISO14443 / FeliCa / etc.)

- No non-volatile EEPROM for user data storage.

So:
👉 It can read, write, or emulate a tag,
but it cannot store permanent tag data like an actual passive NFC sticker (NTAG, MIFARE, etc.) when power is gone.

⚡ 2️⃣ “Reader/Writer” vs. “Card Emulation” in reality

| Mode | PN532 Role | Power Needed | Where data lives |
| --- | --- | --- | --- |
| Reader/Writer | Reads/writes other tags | Yes (active) | In external tag’s memory (like NTAG213) |
| Card Emulation | Pretends to be a tag | Yes (active) | In PN532’s internal RAM buffer, loaded from Arduino each time |

So in Card Emulation mode, when your phone taps it:

- The Arduino loads your message into PN532’s internal buffer via I²C (for example an NDEF record).

- PN532 presents that buffer as a “Type 4 Tag” to the reader.

- The reader (your phone) reads that from the buffer over RF.

- Nothing is stored permanently inside PN532 — when power is lost, it forgets.

🌙 3️⃣ About standby / wake-up behavior

PN532 has a low-power sleep mode and an RF field detection interrupt.
When you put it in card emulation mode:

- It stays partially awake (listening for field).

- When a reader’s field (like a phone NFC antenna) appears nearby,
it detects the RF field and automatically wakes up to respond.

- It then transmits whatever data is sitting in its internal RAM buffer.

So even though your Arduino + PN532 look “idle”, they’re still powered and waiting for RF activation.

🧠 4️⃣ Summary in simple words

- 🔋 Passive tag (sticker) → has EEPROM inside, can hold data forever, no power needed.

- ⚙️ PN532 module → only acts “like” a tag while powered. Data comes from Arduino → loaded into PN532 → read by phone → forgotten on power-off.

- 🌐 When no reader nearby → PN532 sleeps in low-power standby; when phone comes close → RF field wakes it up automatically.

🧩 5️⃣ In your setup

When you connect your Arduino + PN532 over I²C:

- You can tell PN532: “Go into card emulation mode with this text” (for example, your latest sensor line).

- It keeps that text in its small buffer.

- When someone taps a phone → PN532 sends that buffer via RF.

- When you update readings → Arduino overwrites the buffer with new text.

That’s why it behaves like a live-updating virtual tag, not a permanent passive tag.

🧠 What happens in card-emulation mode

- Your Arduino sends the latest sensor data to the PN532 over I²C (or SPI).
→ The PN532 stores that message temporarily in its internal RAM buffer.

- The PN532 stays powered and keeps its RF analog front-end partially active.
→ It continuously listens for any external RF field (like a phone or reader).

- When it detects that field, it automatically wakes fully and answers the reader with the buffer contents.

So yes — the data lives in PN532’s RAM, not flash, and disappears when power is cut.

⚡ Power-use reality

- In card-emulation standby, the PN532 keeps its RF circuitry on (partially).

- It does consume continuous power — roughly 25 mA – 35 mA in this mode (varies by module and antenna tuning).

- When a phone is actively reading, current spikes can reach 50 mA + for a few milliseconds.

So:

- ❌ It’s not low-power like a passive sticker (which uses 0 mA).

- ✅ It’s fine if your Arduino has a steady 5 V supply (USB or adapter).

- ⚠️ But not suitable for battery-only or energy-harvesting devices where every milliamp matters.

🔋 Low-power alternatives

If you truly need “zero-power until read” behavior:

- Use an NFC EEPROM chip like NXP NT3H1101 or NT3H2111.

  - It has a real EEPROM memory accessible both via I²C (Arduino) and RF (phone).

  - The Arduino can write your latest data whenever you log, then go to sleep.

  - Later, any NFC reader can read it without power — the chip harvests energy from RF.

That’s the perfect bridge between your current setup and a real passive tag concept.

✅ Summary

| Feature | PN532 Card Emulation | NTAG I²C (e.g., NT3H1101) |
| --- | --- | --- |
| Needs power | ✅ Yes (always) | ❌ No |
| Data stored in | Volatile RAM | Non-volatile EEPROM |
| Power draw | ~25 – 35 mA | 0 mA (passive) |
| Ideal use | Live, powered system | Batteryless or ultra-low-power logger |

So yes bro — you’re absolutely right:

“Data updated by Arduino in RAM buffer” ✅
“RF field active all the time → consumes more power” ✅

Would you like me to show you how to integrate the NTAG I²C chip as a true passive storage alternative to your PN532?