Report on NFC-Based Low-Power Sensor Data Reading System

1. Introduction / Background

In our project, we use an MCU (Arduino UNO or Mega) connected with multiple sensors, MQ-2, MQ-4, and DHT22 along with a microSD card module.
The system periodically measures sensor values (every 10 minutes) and stores them in a CSV file on the SD card for data logging and later analysis.

However, continuous wireless transmission using Bluetooth or Wi-Fi modules was found to be power-hungry, making them unsuitable for battery-powered or energy-harvesting systems (like solar or electromagnetic recovery setups).
To achieve ultra-low-power real-time data access, we adopted an NFC-based communication interface using the PN532 NFC Module V3.

This module operates at 13.56 MHz (per ISO/IEC 14443 A/B standards) and supports NFC peer-to-peer, card emulation, and reader modes.
It enables reading of latest sensor values directly from the MCU memory without requiring continuous high-power wireless connectivity.

2. Power Efficiency Motivation

According to the PN532 module datasheet (NXP Semiconductors, 2021):

| Parameter | Typical Value | Description / Reference |
| --- | --- | --- |
| Operating voltage | 3.3 – 5 V | Compatible with Arduino UNO/Mega logic levels |
| Operating current (active read mode) | < 100 mA @ 5 V | Active RF field during data transmission |
| Standby current (RF field on) | 10 µA | Ultra-low standby consumption |
| Frequency | 13.56 MHz | ISO/IEC 14443 A/B and FeliCa compliant carrier |
| Communication interface | I²C / SPI / UART | Selectable by DIP switch on V3 board |

In comparison:

- Bluetooth (HC-05 / HM-10): 25–40 mA continuous active current

- Wi-Fi (ESP8266 / ESP32): 70–170 mA transmit current peaks

- NFC (PN532): < 100 mA only during tag read, then drops to < 10 µA idle

Hence, NFC reduces average current draw when data access is infrequent (once per few minutes), making it ideal for low-duty-cycle environmental sensor nodes powered by small batteries or energy harvesters.

3. Overview of NFC Technology

3.1 Principle of Operation

Near-Field Communication (NFC) is a short-range (≤ 10 cm) wireless technology standardized under ISO/IEC 18092 and ECMA-340.
It operates at 13.56 MHz, enabling bidirectional data exchange between:

- a Reader/Writer (Initiator)

- and a Tag/Target (Responder).

NFC supports three operation modes:

- Reader/Writer Mode, Reads/writes passive tags (Type A/B/FeliCa).

- Peer-to-Peer (P2P) Mode, Two NFC devices exchange data (Active ↔ Passive).

- Card Emulation Mode, NFC device behaves like a contactless smart card.

Typical data rates: 106 kbps – 424 kbps, with modulation via amplitude-shift keying (ASK).

Reference: PN532 Datasheet

4. PN532 V3 Module – Description and Features

4.1 Hardware Overview

The PN532 V3 module integrates NXP’s PN532 transceiver IC, an RF antenna, and level-shifting logic for Arduino compatibility.
It exposes SPI, I²C, and UART interfaces selectable by onboard DIP switches.

Key specs from the datasheet:

| Feature | Description |
| --- | --- |
| Chipset | NXP PN532 (ISO/IEC 14443 A/B, 18092) |
| Frequency | 13.56 MHz ± 7 kHz |
| Interface | UART / I²C / SPI selectable |
| Voltage | 3.3 – 5 V tolerant |
| Current | 60 mA active RF, < 10 µA standby |
| Range | Up to 5 cm (depends on antenna & field strength) |

The module’s onboard DIP switch configures the interface:

- SW1 = ON, SW2 = OFF → UART

- SW1 = OFF, SW2 = ON → I²C

- SW1 = OFF, SW2 = OFF → SPI

5. Implementation for Sensor Data Access

5.1 System Architecture

- Sensors → analog/digital inputs to MCU.

- Arduino logs readings to datalog.csv on the SD card every 10 min.

- PN532 connects via SPI/I²C to the same MCU.

- Smartphone or another NFC device reads the latest value from Arduino memory when tapped.

5.2 Method 1: Using a Single NFC Module and NFC-Enabled Mobile Phone

Concept:
The PN532 operates in Card Emulation Mode (Target mode).
The Arduino continuously updates a small NDEF message containing the latest reading.
When a phone is brought near the module, it reads this message as an NFC tag.

Data Flow:

- MCU measures sensors and stores readings:

- Example readings: Temp=27.3°C, Hum=51%, MQ2=143 ppm, MQ4=210 ppm

- MCU formats this as an NDEF Text record.

- PN532 stores and advertises the NDEF data in emulation mode.

- Phone’s NFC reader energizes the field → reads the NDEF payload.

- Android/iOS automatically shows the message.

Advantages:

- No pairing, no app required.

- Ultra-low power (RF only active during tap).

- Secure short-range (≤ 5 cm).

5.3 Method 2: Two NFC Modules (P2P Communication Between MCUs)

Concept:
Use one PN532 as Initiator (Reader/Controller) and another as Target (Sensor Node).
This leverages the Peer-to-Peer Mode defined by NFCIP-1 (ISO/IEC 18092).

Hardware Setup:

- Node A (Sensor Unit): Arduino + Sensors + PN532 (Target)

- Node B (Receiver Unit): Another MCU + PN532 (Initiator)

Process:

- Sensor Node periodically prepares the latest readings.

- When Receiver Node comes near (within 5 cm), it establishes an NFC P2P link.

- The Initiator requests data using “DEP (Data Exchange Protocol)” frames.

- The Target sends back formatted sensor readings (text or binary).

- Receiver displays data on Serial Monitor or LCD.

Advantages:

- Works without smartphones.

- Secure, device-to-device exchange.

- Moderate range (≤ 5 cm).

- Data integrity via CRC16 (per ISO 14443 frame).

5.4 Method 3: Using PN532 as Reader to Poll Passive Tag

In alternate mode, the PN532 acts as a Reader, and a programmable NFC tag (MIFARE Ultralight/NTAG) can store recent readings.
Arduino can update tag memory via I²C at each interval; phone can read tag directly later.
This decouples power domains: the tag remains readable even if MCU is off.

5.5 If Mobile Phone Does Not Support NFC

- Option 1: Use another MCU + PN532 as Initiator (Method 2).

- Option 2: Employ a BLE module (HM-10) temporarily for diagnostics.

- Option 3: Retrieve logs manually via SD card or serial USB connection.

Thus, NFC acts as a primary low-power interface, with BLE/USB serving as fallback for non-NFC devices.

6. Conclusion

The integration of the PN532 V3 NFC module with the sensor-logging MCU system offers a technically efficient and power-optimized solution for real-time data retrieval in portable and autonomous devices.

- NFC’s low activation current and microamp-level standby drastically extend battery life.

- Card Emulation Mode enables instant “tap-to-read” access using any NFC-enabled smartphone.

- Peer-to-Peer Mode provides MCU-to-MCU data transfer without Internet or pairing.

- System remains fully functional even if phones lack NFC support (data available via SD or serial).

This approach aligns with modern low-power IoT design principles, leveraging short-range inductive coupling for minimal-energy, secure, and on-demand communication.